Experiment No: 01

Name of the Experiment: LED Control with STM32 (Blue Pill) Microcontroller

Background Study:

The STM32 (Blue Pill) is a popular and cost-effective microcontroller development board based on the STM32F103C8T6 microcontroller, which belongs to the ARM Cortex-M3 family. This comprehensive experiment covers various LED control techniques, starting with basic blinking, turning an LED on, changing the blink rate, fading the LED brightness, and controlling the LED with a switch. By performing these tasks, you will become familiar with configuring GPIO pins, implementing delays, using PWM for brightness control, and reading input states.

Combined Experiment Procedure:

Part 1: Blinking an LED

Objective: To blink an LED connected to pin PC13 with a fixed interval.

Procedure:

- I have configured GPIO pin PC13 as an output.

- I have set PC13 HIGH to turn on the LED.

- I have waited for one thousand milliseconds.

- I have set PC13 LOW to turn off the LED.

- I have waited for one thousand milliseconds.

- I have repeated the process.

Part 2: Turning on an LED.

Objective: To turn on an LED connected to pin PA15.

Procedure:

- I have configured GPIO pin PA15 as an output.

- I have set PA15 HIGH to turn on the LED.

Part 3: LED with Changing Blink Rate

Objective: To change the blink rate of an LED connected to pin PA15.

Procedure:

- I have configured GPIO pin PA15 as an output.

- I have used a for loop to increase the delay incrementally.

- I have set PA15 HIGH to turn on the LED.

- I have waited for i milliseconds.

- I have set PA15 LOW to turn off the LED.

- I have waited for i milliseconds.

- I have repeated the process with increasing i.

Part 4: Fading the Brightness of an LED

Objective: To fade the brightness of an LED connected to pin PB0.

Procedure:

- I have configured GPIO pin PB0 as an output.

- I have used a for loop to increase the brightness from 0 to 256.

- I have used analogWrite(PB0, i) to set the LED brightness.

- I have delayed for ten milliseconds.

- I have used a for loop to decrease the brightness from 255 to 0.

- I have repeated the process.

Part 5: Controlling an LED with a Switch

Objective: To control an LED connected to pin PB0 using a switch connected to pin PA0.

Procedure:

- I have configured GPIO pin PB0 as an output.

- I have configured GPIO pin PA0 as an input.

- I have read the state of the button connected to PA0.

- If the button state is LOW, I have set PB0 LOW to turn off the LED.

- If the button state is HIGH, I have set PB0 HIGH to turn on the LED.

- I have continuously checked the button state in the loop.

Conclusion:

In this experiment, I have successfully demonstrated various techniques for controlling an LED using the STM32 (Blue Pill) microcontroller. I have configured GPIO pins for output to blink an LED, turn an LED on, change the blink rate, and fade the brightness using PWM. Additionally, I have configured GPIO pins for input to control an LED with a switch. Through these exercises, I have become proficient in using basic GPIO functionality, implementing delays, and utilizing PWM for analog control. This experiment has provided me with practical experience in embedded programming and a solid foundation for more complex projects.